1.

| Wire Color | From (UPS Module) | To (Orange Pi Zero 2W) | Function |
| --- | --- | --- | --- |
| Red | UPS+ (Top Right Pad) | Pin 2 (Physical Pin 2) | +5V Power Input |
| Black | -UPS (Bottom Right Pad) | Pin 6 (Physical Pin 6) | Ground (GND) |

2.

| Wire Color | From (SIM868) | To (Orange Pi Zero 2W) | Function |
| --- | --- | --- | --- |
| Red | VCC (Pin 5) | Pin 2 (Physical Pin 2) | 5V Power Input |
| Black | GND (Pin 4) | Pin 6 (Physical Pin 6) | Ground (GND) |
| Blue | TXD (Pin 3) | Pin 8 (Physical Pin 8) | UART0_TX (Serial Transmit) |
| Blue | RXD (Pin 2) | Pin 10 (Physical Pin 10) | UART0_RX (Serial Receive) |
| Blue | PWREN (Pin 1) | Pin 12 (Physical Pin 12) | GPIO (PH2) (Power Enable Control) |

3.

| Wire Color | From (MPU9250) | To (Orange Pi Zero 2W) | Function |
| --- | --- | --- | --- |
| Red | VCC | Pin 1 | 3.3V Power |
| Blue | SDA | Pin 3 | I2C SDA (Data) |
| Blue | SCL | Pin 5 | I2C SCL (Clock) |
| Black | GND | Pin 9 | Ground (GND) |

4.

| Wire Color | From (MPU6050) | To (Orange Pi Zero 2W) | Function |
| --- | --- | --- | --- |
| Red | VCC | Pin 1 | 3.3V Power |
| Blue | SDA | Pin 3 | I2C SDA (Data) |
| Blue | SCL | Pin 5 | I2C SCL (Clock) |
| Black | GND | Pin 9 | Ground (GND) |

5.

| Wire/Cable | Connection Type | Description |
| --- | --- | --- |
| 24-Pin Ribbon Cable | Interface | Connects the 24 Pin Interface on the Orange Pi Zero 2W to the 24 Pin Interface on the Expansion Board. |

6.

| Wire/Cable | From (Expansion Board USB) | To (DVP-to-USB Bridge) | Function |
| --- | --- | --- | --- |
| Blue Trace 1 | USB 5V | 5V | Power Supply |
| Blue Trace 2 | USB D- | D- | USB Data - |
| Blue Trace 3 | USB D+ | D+ | USB Data + |
| Blue Trace 4 | USB GND | GND | Ground |

| Wire/Cable | From (DVP-to-USB Bridge) | To (OV5640 Camera) | Function |
| --- | --- | --- | --- |
| Ribbon Cable | FPC Connector | FPC Connector | DVP Interface (Video Data) |

7.

| Component | Connected To | Description |
| --- | --- | --- |
| 3.5mm Earphones | Expansion Board Audio Jack | Standard analog audio output for music or system sounds. |